Instr2Op version 1.0 - Written by John Baldwin, 1996

�1Overview

This document provides detailed information on assembling Intel 80x86

assembly instructions, including tables for various addressing methods

and methods of determining operands.  A working knowledge of Intel 80x86

Assembly language is very highly recommended since this is by no means a

tutorial on Assembly language.

�1Help

�2General Help

In this document, section headings are organized as follows:

�1Section Headings

�2Topic Headings

�3Sub-Topic Headings

�2Segment Sizes

In Real Mode, are segments are 16-bit.  In Protected Mode on 80386+, each

segment descriptor has a bit that determines if that selector is a 16-bit

segment or a 32-bit segment.  The size of the segment affects the size of

the operands and the memory references.  If the Address Size or Operand

Size prefixes are used, then the opposite model for addressing and determing

operands is used.  IOW, an instruction in a 32-bit segment that is prefixed

with the Operand Size prefix would execute the instruction as if its

operands were the equivalent 16-bit operands.

�2Opcode Ordering

In Intel 80x86 Assembly, the opcodes follow this pattern in determining the

order of the various opcodes for each instruction:

Code:

->-- Instructions ->--

|                   |

`-------------------'

Instruction:

->-- Prefixes ->-- Instruction ->-- Operands ->-- Data ->---

| |           |      Opcodes     |           | |       |  |

| `----->-----'                  `----->-----' `--->---'  |

|                                                         |

`-------<----------<----------<----------<---------<------'

Prefixes:

->-- Instruction ->-- Address Size ->-- Operand Size ->-- Segment -->--

|     Prefix   | |               | |               | |  Override |

|              | `------->-------' `------->-------' |           |

`------->------'                                     `----->-----'

Instruction Prefix:         Segment Override:

->-- REP ------>--          ->-- CS: -->--

|           |               |       |

|- REPE/Z --|               |- DS: -|

|           |               |       |

|- REPNE/Z -|               |- ES: -|

|           |               |       |

`- LOCK ----'               |- FS: -|

|       |

|- GS: -|

|       |

`- SS: -'

Operands:

->-- Operand -->-- Displacement -->-->--

|  Selector   |               |  |

|             `------->-------'  |

|                                |

`--------------->----------------'

Operand Selector:

->-- Modr/M Byte -->-- SIB -->--

|      |

`--->--'

Address Size     : Uses 32-bit addressing in 16-bit segments, and 16-bit

addressing in 32-bit segments.

Data             : For instructions that use numberical data.  Can be byte,

word, dword, etc. depending on instruction

Instruction      : The opcodes that define which instruction this is.

Memory Offset    : For operands that have memory offsets, they are word or

byte offsets added to memory address operands.

Operand Size     : Uses 32-bit operands in 16-bit segments, and 16-bit

operands in 32-bit segments.

Operands         : The opcodes that define what the parameters are, if any.

Prefixes         : Instruction prefixes.

Segment Override : Force the next instruction to use this segment for any

memory references.

�2Instructions

The instructions are broken up into three main groups: regular instructions,

math co-processor instructions, and protected-mode instructions.  For each

instruction, the first line shall be a sub-topic heading with the

instruction's mnenomic in all caps.  Beside the instruction name will follow

a brief explanation of the instruction- usually the meaning of the mnenomic.

On the next line will be a description of the instruction.  The next line

will give the number of basic opcodes or opcode groups that this instruction

assembles to.  Each opcode or opcode group will be given a sub-topic heading

consisting of the opcode or opcode group number (based on their order in this

document).  Below the heading will be a brief description of the specifics of

the opcode or opcode group.  Beneath the description will be a table that

defines the opcode or opcode group, and below that extra details such as

clock cycles may be appended.  If an opcode or opcode group is defined by a

constant number, then that number may be given in hexadecimal instead of in a

bit-wise table.

�2Prefixes

At the end of the instructions list is a list of the instruction prefixes

such as the segment overrides and the REP prefixes.  Their format basically

follows the same format of the instruction format above.

�2Operand and Address Sizes

Frequently in this document, the symbols OperandSize and AddressSize will

be used in Tables and Instruction Descriptions.  Here are their formal

definitions for this document:

If RealMode Or (ProtectedMode and (SegmentSize=16)) Then

If Address Size Prefix Then

AddressSize = 32

Else

AddressSize = 16

If Operand Size Prefix Then

OperandSize = 32

Else

OperandSize = 16

Else

If Address Size Prefix Then

AddressSize = 16

Else

AddressSize = 32

If Operand Size Prefix Then

OperandSize = 16

Else

OperandSize = 32

�1Tables

See "TABLES.DOC".

�1Instructions

�2Regular Instructions

AAA - ASCII adjust after addition

---------------------------------------------------------------------------

Converts the result of adding two unpacked single-digit BCD numbers (stored

in AL) to an unpacked, single-digit BCD number.

Opcodes: 1

Opcode 1

--------

37

AAD - ASCII adjust before division

---------------------------------------------------------------------------

This sets AL equal to AL + (Immediate byte*AH) and sets AH to 0.  This is

normally used with the immediate byte having a value of 0Ah (10d) to

prepare two unpacked BCD digits for a division operation with an unpacked

result.

Opcodes: 1

Opcode 1

--------

D5

AAM - ASCII adjust after multiplication

---------------------------------------------------------------------------

This sets AH to (AL div Immediate byte) and AL to (AL mod Immediate byte).

Normally, this is used with an immediate byte of 0Ah (10d) to convert the

result of multiplying two unpacked BCD digits in AX.

Opcodes: 1

Opcode 1

--------

D4

AAS - ASCII adjust after subtraction

---------------------------------------------------------------------------

Converts the result of subtracting two unpacked single-digit BCD numbers

(stored in AL) to an unpacked, single-digit BCD number.

Opcodes: 1

Opcode 1

--------

3F

ADC - Add with Carry

---------------------------------------------------------------------------

Adds the two operands and the carry flag together and stores the result in

the first operand.

Opcodes: 3

Opcode 1

--------

Adds a register to a memory reference or another register.

000100xx

^^

���� Operand size:

�       0 - Byte: Use Table 22 to determine operands.

�       1 - Word: Use Table 21 to determine operands.

���� Operand order:

0 - Order of operands same as in tables.

1 - Swap order of operands.

Opcode 2

--------

Adds an immediate to the accumulator register.

1000010x

^

��� Operand size:

0 - Byte: Immediate byte added to AL

1 - Word: Case OperandSize of

16 - Immediate word added to AX

32 - Immediate dword added to EAX

Opcode 3

--------

Adds an immediate to a memory reference or register.

100000xx

^^

���� Operand size:

�       0 - Byte: Immediate byte added to first operand

�                 Operands - Table 24 - Instruction Selector 010

�       1 - Word: Immediate (d)word added to first operand

�                 Operands - Table 23 - Instruction Selector 010

���� Sign-Extension:

0 - Full immediate stored after opcodes

1 - Extend sign of byte after opcodes and use as immediate

ADD - Add

---------------------------------------------------------------------------

Adds two operands together and stores the result in the first operand.

Opcodes: 3

Opcode 1

--------

Adds a register to a memory reference or another register.

000000xx

^^

���� Operand size:

�       0 - Byte: Use Table 22 to determine operands.

�       1 - Word: Use Table 21 to determine operands.

���� Operand order:

0 - Order of operands same as in tables.

1 - Swap order of operands.

Opcode 2

--------

Adds an immediate to the accumulator register.

0000010x

^

��� Operand size:

0 - Byte: Immediate byte added to AL

1 - Word: Case OperandSize of

16 - Immediate word added to AX

32 - Immediate dword added to EAX

Opcode 3

--------

Adds an immediate to a memory reference or register.

100000xx

^^

���� Operand size:

�       0 - Byte: Immediate byte added to first operand

�                 Operands - Table 24 - Instruction Selector 000

�       1 - Word: Immediate (d)word added to first operand

�                 Operands - Table 23 - Instruction Selector 000

���� Sign-Extension:

0 - Full immediate stored after opcodes

1 - Extend sign of byte after opcodes and use as immediate

AND - Logical And

---------------------------------------------------------------------------

Bitwise ANDs the two operands and stores the result in the first operand.

Opcodes: 3

Opcode 1

--------

And a register with a memory reference or another register.

001000xx

^^

���� Operand size:

�       0 - Byte: Use Table 22 to determine operands.

�       1 - Word: Use Table 21 to determine operands.

���� Operand order:

0 - Order of operands same as in tables.

1 - Swap order of operands.

Opcode 2

--------

And an immediate to the accumulator register.

0010010x

^

��� Operand Size:

0 - Byte: Immediate byte ANDed to AL

1 - Word: Case OperandSize of

16 - Immediate word ANDed with AX

32 - Immediate dword ANDed with EAX

Opcode 3

--------

ANDs an immediate with a memory reference or register.

100000xx

^^

���� Operand size:

�       0 - Byte: Immediate byte ANDed with first operand

�                 Operands - Table 24 - Instruction Selector 100

�       1 - Word: Immediate (d)word ANDed with first operand

�                 Operands - Table 23 - Instruction Selector 100

���� Sign-Extension:

0 - Full immediate stored after opcodes

1 - Extend sign of byte after opcodes and use as immediate

BOUND - Check array index against bounds

---------------------------------------------------------------------------

Compares the first operand to two variables of the same size as the first

operand in the memory referenced by the second operand to see if the first

operand is greater than the first variable referenced by the second operand

and less than the second variable.  The variables are stored contiguously.

First the lower bound and then the upper bound.  If the test fails, then

Interrupt 5 is generated with the saved (E)IP pointing to the Bound

instruction that caused the fault.

*Note: "bound reg16,reg16" and "bound reg32,reg32" assemble and seem to be

valid instructions.  However, according to the definition above they

are invalid since a reg16 cannot hold 32 bits and a reg32 cannot hold

64 bits.

Opcodes: 1

Opcode 1

--------

62

Operands - Table 21

CALL - Call a subroutine

---------------------------------------------------------------------------

Call pushes (E)IP (and CS if a far call) onto the stack and jumps to the

specified location.

Opcodes: 3

Opcode 1

--------

Far call to an immediate address following the opcode.

9A

Location -

Case OperandSize of

16: 16-bit offset followed by a 16-bit segment

32: 32-bit offset followed by a 16-bit segment

Case AddressSize of

16: Use CS:IP

32: Use CS:EIP

If AddressSize<>OperandSize Then

If AddressSize=16 Then

High word of 32-bit offset ignored

Else

16-bit offset copied to low word of EIP, high word of EIP

unchanged (?)

Opcode 2

--------

Near call to an immediate (d)word displacement in the current CS.

E8

Opcode 3

--------

Call to a memory reference or a register.

FF

Operands: Table 23 - Instruction Selector 01x

^

��� Target distance:

0: Near

1: Far, only memory

references are valid

(i.e. "call near ax"

is valid, but

"call far ax" is

not.)

CBW, CWDE - Convert byte to word, Convert word to dword.

---------------------------------------------------------------------------

CBW sign extends the value of AL to AX.  CWDE sign extends the value of AX

to EAX.

Opcodes: 1

Opcode 1

--------

98

Case OperandSize of

16: CBW

32: CWDE

CDQ, CWD - Convert dword to qword, Convert word to dword

---------------------------------------------------------------------------

CDQ sign extends the dword in EAX to EDX:EAX.  CWD sign extends the word in

AX to DX:AX.

Opcodes: 1

Opcode 1

--------

99

Case OperandSize of

16: CWD

32: CDQ

CLC - Clear the carry flag

---------------------------------------------------------------------------

Clears the carry flag.

Opcodes: 1

Opcode 1

--------

F8

CLD - Clear the direction flag

---------------------------------------------------------------------------

Clears the direction flag, which is used to control the direction of string

instructions in memory.

Opcodes: 1

Opcode 1

--------

FC

CLI - Clear the interrupt flag

---------------------------------------------------------------------------

Clears the interrupt flag, which disables external interrupts except for

NMI's.

Opcodes: 1

Opcode 1

--------

FA

CMC - Complement carry

---------------------------------------------------------------------------

Toggles the carry flag.

Opcodes: 1

Opcode 1

--------

F5

CMP - Compares two operands

---------------------------------------------------------------------------

Subtracts the second operand from the first operand but throws a way the

result.  It does set the flags accordingly, however.

Opcodes: 3

Opcode 1

--------

Compare a register with a memory reference or another register.

001110xx

^^

���� Operand size:

�       0 - Byte: Use Table 22 to determine operands.

�       1 - Word: Use Table 21 to determine operands.

���� Operand order:

0 - Order of operands same as in tables.

1 - Swap order of operands.

Opcode 2

--------

Compare an immediate with the accumulator register.

0011110x

^

��� Operand Size:

0 - Byte: Immediate byte compared to AL

1 - Word: Case OperandSize of

16 - Immediate word compared with AX

32 - Immediate dword compared with EAX

Opcode 3

--------

Compares an immediate to a memory reference or register.

100000xx

^^

���� Operand size:

�       0 - Byte: Immediate byte compared to first operand

�                 Operands - Table 24 - Instruction Selector 111

�       1 - Word: Immediate (d)word compared to first operand

�                 Operands - Table 23 - Instruction Selector 111

���� Sign-Extension:

0 - Full immediate stored after opcodes

1 - Extend sign of byte after opcodes and use as immediate

CMPSB, CMPSW, CMPSD - Compares memory at DS:(E)SI and ES:(E)DI.

---------------------------------------------------------------------------

Compares either a byte, word, or dword at DS:(E)SI with the same at

ES:(E)DI.  After the compare, the (E)SI and (E)DI registers are updated

by either adding or subtracting 1, 2, or 4 to or from them based on the

value of the direction flag.  If the direction flag is clear, then the

values are added to the registers, otherwise they are subtracted from the

registers.  The flags are set according to the compare.  The order of the

memory references for the purpose of the compare operation is as follows:

cmp [ds:(e)si],[es:(e)di]

Opcodes: 1

Opcode 1

--------

1010011x

^

��� Operand Size:

0 - CMPSB

1 - Case OperandSize of

16 - CMPSW

32 - CMPSD

DAA - Decimal adjust AL after addition

---------------------------------------------------------------------------

Converts the decimal result of adding two packed, two-digit BCD numbers

(stored in AL) into the correct packed, two-digit BCD number.

8Opcodes: 1

Opcode 1

--------

27

DAS - Decimal adjust AL after subtraction

---------------------------------------------------------------------------

Converts the decimal result of subtracting two packed, two-digit BCD

numbers (stored in AL) into the correct packed, two-digit BCD number.

Opcodes: 1

Opcode 1

--------

2F

DEC - Decrement

---------------------------------------------------------------------------

Decrements the operand by 1.

Opcodes: 2

Opcode 1

--------

Decrement (d)word registers.

01001xxx

^^^

����� Register:

Case OperandSize of

16 - Table 4

32 - Table 10

Opcode 2

--------

Decrement a memory reference or register.

1111111x

^

��� Operand size:

0: Byte - Table 24 - Instruction Selector 001

1: Word - Table 23 - Instruction Selector 001

DIV - Unsigned divide

---------------------------------------------------------------------------

Divides the accumulator by the operand and stores the result in the

accumulator.  Bytes are divided into AX with the remainder going into AH

and the quotient into AL.  (D)words are divided into (E)DX:(E)AX with

the remainder going into (E)DX and the quotient going into (E)AX.

Opcodes: 1

Opcode 1

--------

1111011x

^

��� Operand size:

0: Byte - Table 24 - Instruction Selector 110

1: Word - Table 23 - Instruction Selector 110

ENTER - Enter

---------------------------------------------------------------------------

Sets up the stack frame for a subroutine.  It takes two parameters: an

immediate word, followed by an immediate byte.  The word specifies the

amount of stack space to preserve for local variables in the procedure.

The byte designates the nesting level of the subroutine.  This instruction

is equivalent to the following sequence:

push    bp

mov     bp,sp

sub     sp,First Operand (word)

Opcodes: 1

Opcode 1

--------

C8

HLT - Halt

---------------------------------------------------------------------------

Halts the processor until an interrupt or a reset occurs.

Opcodes: 1

Opcode 1

--------

F4

ICEBP, INT01 - ICE Breakpoint

---------------------------------------------------------------------------

This instruction is only useful for ICE (In-Circuit-Emulator) debuggers.

It generates an interrupt 1 as a breakpoint interrupt.  (Interrupt 1 is

usually generated after every instruction whenever the trap flag is set.)

ICEBP was the mnenomic given by an early Intel 80386 ICE, while the P6

documentation gives it the menomic INT01.

Opcodes: 1

Opcode 1

--------

F1

IDIV - Integer divide

---------------------------------------------------------------------------

Divides the accumulator by the operand and stores the result in the

accumulator.  Bytes are divided into AX with the remainder going into AH

and the quotient into AL.  (D)words are divided into (E)DX:(E)AX with

the remainder going into (E)DX and the quotient going into (E)AX.

Opcodes: 1

Opcode 1

--------

1111011x

^

��� Operand size:

0: Byte - Table 24 - Instruction Selector 111

1: Word - Table 23 - Instruction Selector 111

IMUL - Integer multiply

---------------------------------------------------------------------------

Multiplies the third operand by the second operand and stores the result

in the first operand.  If there are only two operands, then the assembler

or debugger is just making the code look neat.  There are really always

three operands, but if the second or third operand is the same as the

first operand, then the second copy of the duplicate operand is omitted

from the instruction when it is displayed as an assembler mnenomic.  In

other words, "imul ax,cx" is really "imul ax,cx,ax".  If there is only one

operand, than that operand is multiplied by the accumulator register

and stored in the accumulator register.  Bytes are multiplied by AL and

stored in AX, and (d)words are multiplied by (E)AX and stored in

(E)DX:(E)AX.

Opcodes: 3

Opcode 1

--------

Multiplies the second operand by an immediate (d)word and stores the result

in the first operand, which must be a (d)word register.

69

Operands - Table 21

Opcode 2

--------

Multiplies the second operand by a sign extended immediate byte and stores

the result in the first operand, which must be a word register.

6B

Operands - Table 21

Opcode 3

--------

Simple one operand integer multiply.

1111011x

^

��� Operand size:

0: Byte - Table 24 - Instruction Selector 101

1: Word - Table 23 - Instruction Selector 101

IN - Read from port.

---------------------------------------------------------------------------

Reads a byte from a port.  The port read from is determined by the second

operand, and the byte read is stored in the first operand.  If the first

operand is a word or dword, then 2 or 4 consecutive ports are read starting

with the second parameter for the lowest byte in the first operand, and the

port second operand+1 for the second lowest byte in the first operand, etc.

Opcodes: 2

Opcode 1

--------

Read data from one of the first 256 ports determined by an immediate byte

following the opcode into the accumulator register.

1110010x

^

��� Operand size:

0 - Read byte from port into AL

1 - Case OperandSize of

16 - Read two ports into AX

32 - Read four ports into EAX

Opcode 2

--------

Read from the port specified by DX into the accumulator register.

1110110x

^

��� Operand size:

0 - Read byte from port DX into AL

1 - Case OperandSize of

16 - Read from two ports into AX

32 - Read from four ports into EAX

INC - Increment

---------------------------------------------------------------------------

Increments the operand by 1.

Opcodes: 2

Opcode 1

--------

Increment (d)word registers.

01000xxx

^^^

����� Register:

Case OperandSize of

16 - Table 4

32 - Table 10

Opcode 2

--------

Increment a memory reference or register.

1111111x

^

��� Operand size:

0: Byte - Table 24 - Instruction Selector 000

1: Word - Table 23 - Instruction Selector 000

INS - Input from port to string

---------------------------------------------------------------------------

The in string instructions read data from port DX and store it at ES:(E)DI

and updates (E)DI based on the size of the data and the direction flag.

If the flag is set, then the size of the data is subtracted from (E)DI.

Otherwise, the data size is added to (E)DI.  Ins cannot be overriden by a

segment override; it always uses ES:(E)DI.

Opcodes: 1

Opcode 1

--------

0110110x

^

��� Data Size: (determines exact instruction)

0: INSB (reads bytes)

1: Case OperandSize of

16 - INSW (reads words)

32 - INSD (reads dwords)

INT - Generate interrupt

---------------------------------------------------------------------------

Calls an interrupt.

Opcodes: 2

Opcode 1

--------

Breakpoint interrupt.  This opcode generates interrupt 3, the breakpoint

interrupt, when it is executed.

CC

Opcode 2

--------

Calls interrupt immediate byte.  I.E. the byte following the opcode

specifies the interrupt to call.

CD

INTO - Call Interrupt on Overflow

---------------------------------------------------------------------------

If the overflow flag is set, then interrupt 4 is called.

Opcodes: 1

Opcode 1

--------

CE

IRET,IRETD - Interrupt return

---------------------------------------------------------------------------

Returns from an interrupt by popping the flags from the stack and

performing a far return.  In protected mode, approx. 300 other things take

place as well, but this is basically a doc on Real Mode (for now), so that

is going to be ignored.

Opcodes: 1

Opcode 1

--------

CF

Case OperandSize of

16 - IRET

32 - IRETD

Jcc - Conditional jumps

---------------------------------------------------------------------------

If condition cc is true then the value of the sign-extended byte or word

following the conditional jump opcode is added to the offset in the CS of

the next instruction after the conditional jump instruction and stored in

IP.

Opcodes: 2

Opcode 1

--------

Conditional jump to a short target (i.e. a byte displacement)

0111xxxx

^^^^

������ Condition - Table 9

Opcode 2

--------

Jump to a short target if (E)CX is zero. (JCXZ/JECXZ)

E3

JMP - Jump

---------------------------------------------------------------------------

Jumps to the target following the opcode.

Opcodes: 4

Opcode 1

--------

Jump to a near target in the current CS.  (Immediate is a displacement, not

an absolute).

E9

Opcode 2

--------

Jump to a far target (i.e. CS:IP).

EA

Opcode 3

--------

Jump to a short target, i.e. 128 bytes backward or 127 bytes forward.

EB

Opcode 4

--------

Near or far jump to a register or memory reference.

FF

Operands: Table 23 - Instruction Selector 10x

^

��� Target distance:

0: Near

1: Far, only memory

references are valid

(i.e. "jmp near ax"

is valid, but

"jmp far ax" is

not.)

LAHF - Load low 8 bits of flag register into AH.

---------------------------------------------------------------------------

Copies the low byte of the flags register into AH.

Opcodes: 1

Opcode 1

--------

9F

LDS - Load into DS

---------------------------------------------------------------------------

Copies the memory referenced by the second parameter into DS and the first

operand.  (The first operand recevies the first (d)word).  If the second

parameter is not a memory reference (e.g. "lds ax,ax") the CPU *will* lock

up unrecoverably, so those are invalid opcodes.

Opcodes: 1

Opcode 1

--------

C5

Operands: Table 21

LEA - Load effective address

---------------------------------------------------------------------------

Stores the offset of the second operand (which must be a memory reference)

in the first operand.  If AddressSize <> OperandSize Then the offset of

the second address will either be zero-extended or only the lowe 16 bits

will be stored.  Although the opcodes for having a register as the second

operand seem to assemble and exist, their validity is questionable.

Opcodes: 1

Opcode 1

--------

8D

Operands - Table 21

LEAVE - Leave

---------------------------------------------------------------------------

Reclaims the stack frame set up by a call to enter.  Leave is equivalent

to the following sequence:

mov    sp,bp

pop    bp

Opcodes: 1

Opcode 1

--------

C9

LES - Load into ES

---------------------------------------------------------------------------

Copies the memory referenced by the second parameter into ES and the first

operand.  (The first operand recevies the first (d)word).  If the second

parameter is not a memory reference (e.g. "les ax,ax") the CPU *will* lock

up unrecoverably, so those are invalid opcodes.

Opcodes: 1

Opcode 1

--------

C4

Operands: Table 21

LODSB, LODSW, LODSD - Copy from DS:(E)SI to the accumulator register.

---------------------------------------------------------------------------

Copies either a byte, word, or dword from DS:(E)SI to the accumulator

register (AL, AX, or EAX).  Afterwards, it updates (E)SI by either adding

or subtracting 1, 2, or 4 to or from (E)SI based on the direction flag.

If the direction flag is set, then the value is subtracted from (E)SI,

otherwise the value is added to (E)SI.

Opcodes: 1

Opcode 1

--------

1010110x

^

��� Operand size:

0 - LODSB

1 - Case OperandSize of

16 - LODSW

32 - LODSD

LOOP, LOOPZ, LOOPNZ - Loop, Loop While Zero, Loop While Not Zero

---------------------------------------------------------------------------

Loop decrements the counter register, (E)CX, and if it is zero, it jumps

to the code referenced by the immediate byte offset following it.

Opcodes: 2

Opcode 1

--------

Loop until counter is zero or the zero flag is set or clear.

1110000x

^

��� Zero state:

0 - LOOPZ

1 - LOOPNZ

Opcode 2

--------

Loop until counter is zero.

E2

MOV - Move

---------------------------------------------------------------------------

Copies the value of the second operand into the first operand.

Opcodes: 6

Opcode 1

--------

Move a register into a memory reference or another register and vice versa.

100010xx

^^

���� Operand size:

�       0 - Byte: Use Table 22 to determine operands.

�       1 - Word: Use Table 21 to determine operands.

���� Operand order:

0 - Order of operands same as in tables.

1 - Swap order of operands.

Opcode 2

--------

Move a segment register into a memory reference or another register and

vice versa.

100011x0

^

���� Operand order:

0 - Order of operand same as in tables.

1 - Swap order of operands.

Operands - Table 26

Opcode 3

--------

Move a direct memory reference (i.e. no registers) into the accumulator

register.

101000xx

^^

���� Operand size:

�       0 - Byte operands

�       1 - (D)word operands

���� Operand order:

0 - AL/AX/EAX first operand

1 - AL/AX/EAX second operand

Opcode 4

--------

Move an immediate byte into an 8-bit register.

10110xxx

^^^

����� Destination register - Table 3

Opcode 5

--------

Move an immediate (d)word into a (d)word register.

10111xxx

^^^

����� Destination register:

Case OperandSize of

16 - Table 4

32 - Table 10

Opcode 6

--------

Move an immediate byte or (d)word into a memory reference or register.

1100011x

^

��� Operand size:

0 - Byte - Table 24 - Instruction Selector xxx (i.e. anything)

1 - Word - Table 23 - Instruction Selector xxx (i.e. anything)

MOVSB, MOVSW, MOVSD - Copies memory from DS:(E)SI to ES:(E)DI.

---------------------------------------------------------------------------

The movs string instructions copy a byte, word, or dword of memory from

DS:(E)SI to ES:(E)DI.  Afterwards, they either add or subtract 1, 2, or 4

to or from (E)SI and (E)DI based on the value of the direction flag.

If the direction flag is set, then the values are subtracted from the

registers, otherwise the values are added to the registers.

Opcodes: 1

Opcode 1

--------

1010010x

^

��� Operand size:

0 - MOVSB

1 - Case OperandSize of

16 - MOVSW

32 - MOVSD

MUL - Unsigned multiply

---------------------------------------------------------------------------

Multiplies the operand by the accumulator register and stores the result

in the accumulator register.  Bytes are multiplied by AL and stored in AX.

(D)words are multiplied by (E)AX and stored in (E)DX:(E)AX.

Opcodes: 1

Opcode 1

--------

1111011x

^

��� Operand size:

0: Byte - Table 24 - Instruction Selector 100

1: Word - Table 23 - Instruction Selector 100

NEG - Negate

---------------------------------------------------------------------------

Sets the operand to the 2's complement of itself.  Basically toggles the

sign of a number.  (Not by just toggling the sign bit, however).

NEG is the same as the following:

not <operand>

inc <operand>

Opcodes: 1

Opcode 1

--------

1111011x

^

��� Operand size:

0: Byte - Table 24 - Instruction Selector 011

1: Word - Table 23 - Instruction Selector 011

NOP - No operation

---------------------------------------------------------------------------

NOP is actually the opcode for "xchg ax,ax".

Opcodes: 1

Opcode 1

--------

90

NOT - Not

---------------------------------------------------------------------------

Sets the operand equal to the 1's complement of itself.  It toggles every

bit in the operand.

Opcodes: 1

Opcode 1

--------

1111011x

^

��� Operand size:

0: Byte - Table 24 - Instruction Selector 010

1: Word - Table 23 - Instruction Selector 010

OR - Logical or

---------------------------------------------------------------------------

Bitwise ORs the two operands and stores the result in the first operand.

Opcodes: 3

Opcode 1

--------

Or a register with a memory reference or another register.

000010xx

^^

���� Operand size:

�       0 - Byte: Use Table 22 to determine operands.

�       1 - Word: Use Table 21 to determine operands.

���� Operand order:

0 - Order of operands same as in tables.

1 - Swap order of operands.

Opcode 2

--------

Or an immediate to the accumulator register.

0000110x

^

��� Operand Size:

0 - Byte: Immediate byte ORed to AL

1 - Word: Case OperandSize of

16 - Immediate word ORed with AX

32 - Immediate dword ORed with EAX

Opcode 3

--------

ORs an immediate with a memory reference or register.

100000xx

^^

���� Operand size:

�       0 - Byte: Immediate byte ORed with first operand

�                 Operands - Table 24 - Instruction Selector 001

�       1 - Word: Immediate (d)word ORed with first operand

�                 Operands - Table 23 - Instruction Selector 001

���� Sign-Extension:

0 - Full immediate stored after opcodes

1 - Extend sign of byte after opcodes and use as immediate

OUT - Output data to port

---------------------------------------------------------------------------

Writes the second operand to the port specified by the first operand.

If the second operand is a word or dword, then the 2 or 4 bytes in the

second operand are written to either 2 or 4 consecutive ports starting

with the first operand.

Opcodes: 2

Opcode 1

--------

Output the accumulator register to one of the first 256 ports referenced by

an immediate byte following the opcode.

1110011x

^

��� Operand size:

0 - Output AL to the port

1 - Case OperandSize of

16 - Output AX to two ports

32 - Output EAX to four ports

Opcode 2

--------

Output the accumulator register to the port specified by DX.

1110111x

��� Operand size:

0 - Output AL to port DX

1 - Case OperandSize of

16 - Output AX to two ports

32 - Output EAX to four ports

OUTS - Output to port from string

---------------------------------------------------------------------------

The out string instructions read data from DS:(E)SI and out it to port DX.

They also update (E)SI based on the size of the data and the direction flag.

If the flag is set, then the size of the data is subtracted from (E)SI.

Otherwise, the data size is added to (E)SI.  The segment for outs cannot be

overridden; data is always read from DS:(E)SI.

Opcodes: 1

Opcode 1

--------

0110111x

^

��� Data Size: (determines exact instruction)

0: OUTSB (reads bytes)

1: Case OperandSize of

16 - OUTSW (reads words)

32 - OUTSD (reads dwords)

POP - Pop data off the stack

---------------------------------------------------------------------------

Copies the data pointed to by SS:SP into the parameter and unallocates

the data copied from the top of the stack.

Opcodes: 3

Opcode 1

--------

Pop various segment registers from the stack.

000xx111

^^

������� Segment register:

00 - ES

10 - SS

11 - DS

Opcode 2

--------

Pops (d)word registers from the stack.

01011xxx

^^^

����� Register:

Case OperandSize of

16 - Table 4

32 - Table 10

Opcode 3

--------

Pops (d)word registers or memory references from the stack.

8F

Operands - Table 23 - Instruction Selector xxx (all are same instruction)

POPA, POPAD - Pop all general purpose registers from the stack.

----------------------------------------------------------------------------

Pops the 8 16- or 32-bit general registers from the stack in the following

order: DI, SI, BP, SP (discarded), BX, DX, CX, and AX.  If the OperandSize =

32, then the 32-bit versions of the registers are popped instead.

Opcodes: 1

Opcode 1

--------

61

Case OperandSize of

16: POPA

32: POPAD

POPF, POPFD - Pop the flags register from the stack.

---------------------------------------------------------------------------

Pops either the 16-bit (POPF) or 32-bit (POPFD) flags register from the

stack.

Opcodes: 1

Opcode 1

--------

9D

Case OperandSize of

16: POPF

32: POPFD

PUSH - Push data onto the stack

---------------------------------------------------------------------------

Allocates space on the stack and stores the parameter there.

Opcodes: 5

Opcode 1

--------

Pushes various segment registers onto the stack.

000xx110

^^

������� Segment register:

00 - ES

01 - CS

10 - SS

11 - DS

Opcode 2

--------

Pushes (d)word registers onto the stack.

01010xxx

^^^

����� Register:

Case OperandSize of

16 - Table 4

32 - Table 10

Opcode 3

--------

Pushes an immediate (d)word onto the stack.

68

Opcode 4

--------

Pushes a sign extended immediate byte onto the stack as a (d)word.

6A

Opcode 5

--------

Pushes a (d)word memory reference or register onto the stack.

FF

Operands: Table 23 - Instruction Selector 110

PUSHA, PUSHAD - Push all general purpose registers onto the stack.

----------------------------------------------------------------------------

Pushes the 8 16- or 32-bit general registers onto the stack in the following

order: AX, CX, DX, BX, SP (before PUSHA instruction), BP, SI, and DI.  If

the OperandSize = 32, then the 32-bit versions of the registers are pushed

instead.

Opcodes: 1

Opcode 1

--------

60

Case OperandSize of

16: PUSHA

32: PUSHAD

PUSHF, PUSHFD - Push the flags register

----------------------------------------------------------------------------

Pushes either the 16-bit (PUSHF) or the 32-bit (PUSHFD) flags register onto

the stack.

Opcodes: 1

Opcode 1

--------

9C

Case OperandSize of

16: PUSHF

32: PUSHFD

RCL - Rotate through carry left.

----------------------------------------------------------------------------

Rotates the bits in the first operand and the carry flag left by counter

times.  At each rotation, the bits in the bit are all shifted left.  The

carry flag is copied to the LSB of the first operand, and the original MSB

of the first operand is copied to the carry flag.

Opcodes: 2

Opcode 1

--------

Rotate first operand left by an immediate number of times.

1100000x

^

��� Operands:

0 - Table 23 - Instruction Selector 010

1 - Table 24 - Instruction Selector 010

Opcode 2

--------

Rotate first operand left once or by CL times.

110100xx

^^

���� Operands:

�       0 - Table 23 - Instruction Selector 010

�       1 - Table 24 - Instruction Selector 010

���� Count:

0 - 1 time

1 - CL times

RCR - Rotate through carry right.

----------------------------------------------------------------------------

Rotates the bits in the first operand and the carry flag right by counter

times.  At each rotation, the bits in the bit are all shifted right.  The

carry flag is copied to the MSB of the first operand, and the original LSB

of the first operand is copied to the carry flag.

Opcodes: 2

Opcode 1

--------

Rotate first operand right by an immediate number of times.

1100000x

^

��� Operands:

0 - Table 23 - Instruction Selector 011

1 - Table 24 - Instruction Selector 011

Opcode 2

--------

Rotate first operand right once or by CL times.

110100xx

^^

���� Operands:

�       0 - Table 23 - Instruction Selector 011

�       1 - Table 24 - Instruction Selector 011

���� Count:

0 - 1 time

1 - CL times

RET, RETF, RETN - Return from subroutine call.

----------------------------------------------------------------------------

Pops (E)IP (and CS if it is a far return) off of the stack.

Opcodes: 4

Opcode 1

--------

Discard immediate word number of bytes from the stack and then return near.

C2

Opcode 2

--------

Return near.

C3

Opcode 3

--------

Discard immediate word number of bytes from the stack and then return far.

CA

Opcode 4

--------

Return far.

CB

ROL - Rotate left.

----------------------------------------------------------------------------

Rotates the bits in the first operand left by counter times.  After the

rotation, the carry flag is the same as the LSB, as that was the last bit

rotated.

Opcodes: 2

Opcode 1

--------

Rotate first operand left by an immediate number of times.

1100000x

^

��� Operands:

0 - Table 23 - Instruction Selector 000

1 - Table 24 - Instruction Selector 000

Opcode 2

--------

Rotate first operand left once or by CL times.

110100xx

^^

���� Operands:

�       0 - Table 23 - Instruction Selector 000

�       1 - Table 24 - Instruction Selector 000

���� Count:

0 - 1 time

1 - CL times

ROR - Rotate right.

----------------------------------------------------------------------------

Rotates the bits in the first operand right by counter times.  After the

rotation, the carry flag is the same as the MSB, as that was the last bit

rotated.

Opcodes: 2

Opcode 1

--------

Rotate first operand right by an immediate number of times.

1100000x

^

��� Operands:

0 - Table 23 - Instruction Selector 001

1 - Table 24 - Instruction Selector 001

Opcode 2

--------

Rotate first operand right once or by CL times.

110100xx

^^

���� Operands:

�       0 - Table 23 - Instruction Selector 001

�       1 - Table 24 - Instruction Selector 001

���� Count:

0 - 1 time

1 - CL times

SAL, SHL - Shift left.

----------------------------------------------------------------------------

Shifts the bits in the first operand left by counter times.  At each

rotation, the MSB of the first operand is copied to the carry flag, the bits

in the first operand are shifted left, and the LSB of the first operand is

set to 0.

Opcodes: 2

Opcode 1

--------

Shift first operand left by an immediate number of times.

1100000x

^

��� Operands:

0 - Table 23 - Instruction Selector 100

1 - Table 24 - Instruction Selector 100

Opcode 2

--------

Shift first operand left once or by CL times.

110100xx

^^

���� Operands:

�       0 - Table 23 - Instruction Selector 100

�       1 - Table 24 - Instruction Selector 100

���� Count:

0 - 1 time

1 - CL times

SALC - Set AL on Carry

----------------------------------------------------------------------------

This sets all the bits in AL to the value of the carry flag.  So, if the

carry flag is set, AL is set to FF and it the carry flag is clear, AL is set

to 0.

(Note: This instruction is undocumented, but it does work.)

Opcodes: 1

Opcode 1

--------

D6

SAR - Shift arithmetic right.

----------------------------------------------------------------------------

Shifts the bits in the first operand right by counter times.  At each

rotation, the LSB of the first operand is copied to the carry flag, the bits

in the first operand are shifted right, and the MSB of the first operand is

unchanged to preserve the sign of signed values.

Opcodes: 2

Opcode 1

--------

Shift first operand right by an immediate number of times.

1100000x

^

��� Operands:

0 - Table 23 - Instruction Selector 111

1 - Table 24 - Instruction Selector 111

Opcode 2

--------

Shift first operand right once or by CL times.

110100xx

^^

���� Operands:

�       0 - Table 23 - Instruction Selector 111

�       1 - Table 24 - Instruction Selector 111

���� Count:

0 - 1 time

1 - CL times

SAHF - Copy AH to the low 8 bits of the flags register.

----------------------------------------------------------------------------

Copies the value of AH to the low byte of the flags register.

Opcodes: 1

Opcode 1

--------

9E

SBB - Subtract with borrow

----------------------------------------------------------------------------

Subtracts the sum of the second operand and the carry flag from the first

operand and stores the result in the first operand.

Opcodes: 3

Opcode 1

--------

Subtract a register from a memory value or a register.

000110xx

^^

���� Operand size:

�       0 - Byte: Use Table 22 to determine operands.

�       1 - Word: Use Table 21 to determine operands.

���� Operand order:

0 - Order of operands same as in tables.

1 - Swap order of operands.

Opcode 2

--------

Subtract an immedate from the accumulator register.

0001110x

^

��� Operand Size:

0 - Byte: Immediate byte subtracted from AL

1 - Word: Case OperandSize of

16 - Immediate word subtracted from AX

32 - Immediate dword subtracted from EAX

Opcode 3

--------

Subtracts an immediate from a memory reference or register.

100000xx

^^

���� Operand size:

�       0 - Byte: Immediate byte subtracted from first operand

�                 Operands - Table 24 - Instruction Selector 011

�       1 - Word: Immediate (d)word subtracted from first operand

�                 Operands - Table 23 - Instruction Selector 011

���� Sign-Extension:

0 - Full immediate stored after opcodes

1 - Extend sign of byte after opcodes and use as immediate

SCASB, SCASW, SCASD - Compare the accumulator register with ES:(E)DI.

----------------------------------------------------------------------------

Compares the value of AL, AX, or EAX, with the byte, word, or dword at

ES:(E)DI and sets the flags accordingly.  Afterwards, (E)DI is updated by

either adding or subtracting 1, 2, or 4 bytes to or from (E)DI based on the

the value of the direction flag.  If the direction flag is set, the value

is subtracted from (E)DI, otherwise, the value is added to (E)DI.  For the

purposes of the comparison, the operands are ordered as follows:

cmp al,[es:(e)di]

Opcodes: 1

Opcode 1

--------

1010111x

^

��� Operand size:

0 - SCASB

1 - Case OperandSize of

16 - SCASW

32 - SCASD

SHR - Shift right.

----------------------------------------------------------------------------

Shifts the bits in the first operand right by counter times.  At each

rotation, the LSB of the first operand is copied to the carry flag, the bits

in the first operand are shifted right, and the MSB of the first operand is

set to 0.

Opcodes: 2

Opcode 1

--------

Shift first operand right by an immediate number of times.

1100000x

^

��� Operands:

0 - Table 23 - Instruction Selector 101

1 - Table 24 - Instruction Selector 101

Opcode 2

--------

Shift first operand right once or by CL times.

110100xx

^^

���� Operands:

�       0 - Table 23 - Instruction Selector 101

�       1 - Table 24 - Instruction Selector 101

���� Count:

0 - 1 time

1 - CL times

STC - Set the carry flag

---------------------------------------------------------------------------

Sets the carry flag.

Opcodes: 1

Opcode 1

--------

F9

STD - Set the direction flag

---------------------------------------------------------------------------

Sets the direction flag, which is used to control the direction of string

instructions in memory.

Opcodes: 1

Opcode 1

--------

FD

STI - Set the interrupt flag

---------------------------------------------------------------------------

Sets the interrupt flag, which enables external interrupts.

Opcodes: 1

Opcode 1

--------

FB

STOSB, STOSW, STOSD - Copy the accumulator register to ES:(E)DI.

----------------------------------------------------------------------------

Copies the value of AL, AX, or EAX (depending on the instruction) to

ES:(E)DI.  Afterwards, it updates (E)DI by adding or subtracting 1, 2, or 4

to or from (E)DI based on the direction flag.  If the direction flag is set,

the values are subtracted from (E)DI, otherwise they are added to (E)DI.

Opcodes: 1

Opcode 1

--------

1010101x

^

��� Operand size:

0 - STOSB

1 - Case OperandSize of

16 - STOSW

32 - STOSD

SUB - Subtract

----------------------------------------------------------------------------

Subtracts the the second operand from the first operand and stores the

result in the first operand.

Opcodes: 3

Opcode 1

--------

Subtract a register from a memory value or a register.

001010xx

^^

���� Operand size:

�       0 - Byte: Use Table 22 to determine operands.

�       1 - Word: Use Table 21 to determine operands.

���� Operand order:

0 - Order of operands same as in tables.

1 - Swap order of operands.

Opcode 2

--------

Subtract an immediate from the accumulator register.

0010110x

^

��� Operand Size:

0 - Byte: Immediate byte subtracted from AL

1 - Word: Case OperandSize of

16 - Immediate word subtracted from AX

32 - Immediate dword subtracted from EAX

Opcode 3

--------

Subtracts an immediate from a memory reference or register.

100000xx

^^

���� Operand size:

�       0 - Byte: Immediate byte subtracted from first operand

�                 Operands - Table 24 - Instruction Selector 101

�       1 - Word: Immediate (d)word subtracted from first operand

�                 Operands - Table 23 - Instruction Selector 101

���� Sign-Extension:

0 - Full immediate stored after opcodes

1 - Extend sign of byte after opcodes and use as immediate

TEST - Test to see if bits are set

---------------------------------------------------------------------------

Performs a bitwise AND on the two operands and discards the result.

Opcodes: 3

Opcode 1

--------

Test bits in a register with a memory reference or register.

1000010x

^

��� Operand size:

0: Byte - See Table 22 for operands.

1: Word - See Table 21 for operands.

Opcode 2

--------

Test bits in the accumulator register with an immediate.

1010100x

^

��� Operand size:

0: Byte - use AL

1: Word - use (E)AX

Opcode 3

--------

Test bits in a memory reference or register with an immediate.

1111011x

^

��� Operand size:

0: Byte - Table 24 - Instruction Selector 000

1: Word - Table 23 - Instruction Selector 000

WAIT - Wait until BUSY# pin is inactive

---------------------------------------------------------------------------

Wait halts the CPU until the BUSY# pin becomes high.  The BUSY# pin is

controlled by the math co-processor.

Opcodes: 1

Opcode 1

--------

9B

XCHG - Exchange values

---------------------------------------------------------------------------

Exchanges the values of the two operands.

Opcodes: 2

Opcode 1

--------

Exchange a register with a memory reference or another register.

1000011x

^

��� Operand Size:

0: Byte - See Table 22 for operands.

1: Word - See Table 21 for operands.

Opcode 2

--------

Exchange a (d)word register with the accumulator.

10010xxx

^^^

����� Register to exchange with (E)AX:

Case OperandSize of

16 - Table 4

32 - Table 10

XLAT - Table lookup translation

---------------------------------------------------------------------------

This can be used to read values from a table., basically, this instruction

sets AL to the byte of memory at DS:[(E)BX+ unsigned AL].

Opcodes: 1

Opcode 1

--------

D7

XOR - Logical exclusive or

---------------------------------------------------------------------------

Bitwise exclusive ORs the two operands and stores the result in the first

operand.

Opcodes: 3

Opcode 1

--------

XORs a register with a memory reference or another register.

001100xx

^^

���� Operand size:

�       0 - Byte: Use Table 22 to determine operands.

�       1 - Word: Use Table 21 to determine operands.

���� Operand order:

0 - Order of operands same as in tables.

1 - Swap order of operands.

Opcode 2

--------

XORs an immediate with the accumulator register.

0011010x

^

��� Operand Size:

0 - Byte: Immediate byte XORed with AL

1 - Word: Case OperandSize of

16 - Immediate word XORed with AX

32 - Immediate dword XORed with EAX

Opcode 3

--------

XORs an immediate with a memory reference or register.

100000xx

^^

���� Operand size:

�       0 - Byte: Immediate byte XORed with first operand

�                 Operands - Table 24 - Instruction Selector 110

�       1 - Word: Immediate (d)word XORed with first operand

�                 Operands - Table 23 - Instruction Selector 110

���� Sign-Extension:

0 - Full immediate stored after opcodes

1 - Extend sign of byte after opcodes and use as immediate

Math Co-Processor Instructions

==============================

Protected Mode Instructions

===========================

ARPL - Adjust RPL (Requested Priviledge Level) field of selector.

---------------------------------------------------------------------------

The first operand is a 16-bit memory reference or register that holds the

value of a selector.  The second operand is a word register.  If the RPL

field of the first operand (the lower two bits) is less than the RPL field

of the second operand, then ZF = 1 and the RPL of the first operand is

set to the RPL of the second operand.  Otherwise ZF = 0 and the first

operand isn't changed.

This can be used by the operating system to insure that callers to routines

do not have a higher RPL than they should.

*Note: This instruction only takes 16-bit operands according to the

documentation from TASM.  Apparently, however, it does use 32-bit

operands.  However, this makes no sense since it deals with

selectors, which are only 16 bits wide.

Opcodes: 1

Opcode 1

--------

63

Operands - Table 21

����������Ŀ

� Prefixes �

������������

LOCK - Lock the bus

---------------------------------------------------------------------------

Causes the LOCK# signal of the CPU to be asserted during the next

instruction.  This instruction is only useful in multiple processor

environments, where it can be used to assure that the CPU has exclusive

access to any shared memory.

Opcodes: 1

Opcode 1

--------

F0

Segment Overrides

---------------------------------------------------------------------------

Forces the processor to use the segment represented by the override to be

used in all memory references for the next instruction.

Opcodes: 2

Opcode 1

--------

Segment overrides for ES, DS, CS, and SS.

001xx110

^^

������� Segment register:

00 - ES

01 - CS

10 - SS

11 - DS

Opcode 2

--------

Segment overrides for FS and GS

011001xx

^^

���� Segment register:

00 - FS

01 - GS

1x - most likely reserved for two more segment registers

REP, REPZ, REPNZ

---------------------------------------------------------------------------

Repeats the following string instruction (CMPS, INS, LODS, MOVS, OUTS,

SCAS, STOS) (E)CX times while the specified condition of the zero flag

is met (CMPS, SCAS only).  REP is used with INS, LODS, MOVS, OUTS, and

STOS.  REPZ and REPNZ (and their aliases REPE and REPNE) are used with CMPS

and SCAS.

Opcodes: 1

Opcode 1

--------

1111001x

^

��� ZF state:

0 - REPNZ

1 - REP, REPZ

===========================================================================

Copyright 1996, John Baldwin.

The author wishes to acknowledge the following for their assistance:

Robert Collins - various documents regarding undocumented opcodes.

_Turbo_Assembler_Quick_Reference_Guide_.  Borland International: Scott

Valley, 1991.

Swan, Tom.  _Mastering_Turbo_Assember_.  Sams Publishing: Indianapolis,

1995.